## 387

Research Article

Available Online at: www.ijpir.com

# International Journal of Pharmacy and Industrial Research

ISSN

Print	2231 – 3648

Online	2231 – 3656

DEVELOPMENT AND VALIDATION OF SPECTROSCOPIC METHOD FOR THE ESTIMATION OF NAFTOPIDIL IN BULK AND DOSAGE FORM

## *1M.Sunil kumar, 2M.Swapna, 3V.Kirankumar, 4K.Priyanka,5M.Manasa

*1,4,5Srikakathiya Institute of Pharmaceutical Sciences, Hanamkkonda, Warangal, A.P. India.

2Talla Padmavathi college of Pharmacy, Orus, Warangal, A.P. India.

3Unity college of pharmacy, Raigir, Bhongir, Nalgonda, A.P. India

# Abstract

A simple, rapid and sensitive method has been developed for the quantitative estimation of n in bulk and tablet. The wavelength 285 nm was selected for the estimation of drug using distilled water: acetonitrile (50:50) as a solvent. The drug obeyed Beer-lambert’s law over the concentration range 5-25g/ml. The accuracy of the method was assessed by recovery studies and was found between 99.73-100.17%. The method was statistically validated for the linearity, precision, accuracy, repeatability, LOD, LOQ and ruggedness. The method was successfully applied for routine analysis of this drug in bulk and formulations.

Key words: Naftopidil, UV-spectroscopy, Validation, Recovery

# Introduction

Naftopidil,a phentermine derivative, a selective α1- adreno receptor antagonist, a calcium antagonist and a 5-HT1A agonist1. It is a renal urological drug that is utilized extensively for the treatment of arterial hypertension and benign prostatic hypertrophy (BPH) 2. Literature survey reveals that few chiral HPLC3, 4, RP-HPLC in bio samples 5-8 and phosphorimetric methods9, 10 reported for the estimation of naftopidil. No simple UV- spectroscopic method has been reported for the estimation in dosage forms. In the present study to develop a simple, accurate and precise UV- spectroscopic method for estimation of naftopidil in bulk and dosage form. The validation has been carried out as per ICH guidelines.

# Experimental

### Apparatus

A shimazu-1700 double beam UV-Visible Spectrophotometer with 1 cm matched quartz cell was used for all spectral measurements.

### Materials

All the chemicals used were of analytical grade. A gift sample of Naftopidil obtained from Hetero Laboratories, Hyderabad, India was used as working standard. The formulation (Naftomax- 50mg) was purchased from local pharmacy.

Preparation of standard stock solution Standard	stock	solution	was	prepared	by dissolving 100 mg of Naftopidil in the distilled

Author for Correspondence:

M.Sunil kumar,

Srikakathiya Institute of Pharmaceutical Sciences, Hanamkkonda, Warangal, A.P. India.

Email: ravi_sunil85@yahoo.com

388

M.Sunil kumar. et al., Int. J. Pharm & Ind. Res., Vol.–02 (04) 2012 [387 - 389]

water:acetonitrile (50:50) and the volume was adjusted to 100ml with the same, to give a solution of concentration 1000µg/ml. Different aliquots were transferred to series of 100ml volumetric flasks and volume was made up to the mark with distilled water: acetonitrile (50:50) to obtain series of concentrations. The absorbances of these solutions were measured at 285 nm and a calibration curve was constructed, by plotting the absorbance against the corresponding drug concentration.

### Assay procedure

To determine the content of Naftopidil in solid dosage form, twenty tablets containing the drug were accurately weighed. Their average weight was determined and finely powdered. An amount of powder, equivalent to 25 mg of Naftopidil was weighed, dissolved in distilled water: acetonitrile (50:50) and shaken mechanically for 20min. The solution was filtered and diluted to 25ml with distilled water: acetonitrile (50:50). Aliquot of 1ml from this solution was further diluted ten times its volume with distilled water: acetonitrile (50:50) and scanned at 285 nm. The procedure was repeated six times.

# Results and discussion

The drug Naftopidil was analyzed at 285nm in distilled water:acetonitrile (50:50) using UV- Visible spectrophotometer. Optical characteristics such as Beers's law limits, intercept and slope has been calculated using regression equation, which has been presented in Table 1.

Intra-day and inter-day precision: precision was determined by analyzing the drug at three different concentrations and each concentration for three times, on the same day. Inter-day precision was determined similarly, analyzing the samples daily, for three consecutive days. The results are summarised in Table 2.

To ensure the accuracy of method, recovery studies were performed by standard addition method at 50%, 75% and100% levels of drug concentration, to the pre-analyzed samples and percent recovery values were calculated. Recovery experiment indicated the absence	of	interferences	from	the	commonly encountered pharmaceutical additives and excipients. Sensitivity of the method was determined in terms of limit of detection (LOD) and limit of quantification (LOQ). The LOD and LOQ were calculated by using the formula, LOD=3.3σ/S and LOQ=10σ/S, where  is residual standard deviation of the regression line and S is the slope of the corresponding regression line. The LOD and LOQ were found to be 0.03387 g/ml and 0 .10264 g/ml. The results did not show any statistical difference between operators and an instrument suggesting that method developed was rugged.

The developed analytical method was validated10 as per the guidelines laid by USP. The developed method was found to be simple, accurate, precise, sensitive and economical. The results of the validation tests were found to be satisfactory and therefore, this method can be applied to routine analyze of drug in formulations.

# Table No. 01: Optical characteristics of Naftopidil

| Parameters | Values |
| --- | --- |
| max (nm) | 285 |
| Beers law limit (g/ml) | 5-25 |
| Correlation coefficient (r) | 0.999 |
| Regression equation (y=mx+c) | Y=0.027X+0.023 |
| Slope(m) | 0.027 |
| Intercept(c) | 0.023 |
| LOD (g/ml) | 0.03387 |
| LOQ (g/ml) | 0.10264 |
| Standard error | 0.01402 |

M.Sunil kumar. et al., Int. J. Pharm & Ind. Res., Vol.–02 (04) 2012 [387 - 389]

# Table No. 02: Results of assay, recovery, and precision and ruggedness data

| Parameters | Results |
| --- | --- |
| Amount found ( mg/ Tab) | 50.185 |
| % Recovery ( n=3) | 99.732 |
| % RSD | 0.03337 |
| Precision [% RSD] |  |
| Intra –day (n=3) | 0.44889 |
| Inter-day(n=3) | 0.41281 |
| Repeatability(n=6) | 0.0501 |
| Ruggedness [% RSD] |  |
| Analyst I (n=3) | 0.0511 |
| Analyst II ( n=3) | 0.0505 |
| Instrument I ( n=3) | 0.0495 |
| Instrument II ( n=3) | 0.0470 |

Acknowledgement

The	authors	are	thankful	to	Hetero	Labs, Hyderabad for the gift sample of Naftopidil.

# References

- J.M.Beale; Textbook of Organic Medicinal and Pharmaceutical chemistry, 10th Ed., Lippincott Williams and Wilkins, 367-378 (2004).

- S.Budawari; The Merck index, 13th Ed., Merck and co.inc., U.S.A.,(2001).

- C.Sweetman; Martindale- the Complete Drug Reference, 33rd Ed., Pharmaceutical Press, London, (2002).

- D.A.Williams. T.L. Lemke; Foye’s Principles of Medicinal Chemistry, 5th Ed., Lippincott Williams and Wilkins, Wolters Wuwer Health Pvt. Ltd., New Delhi,(2006).

- Katzung; Basic and Clinical Pharmacology, 9th Ed., Mc raw Hill, (2004).

- H.P. Range, M. M. Dale, J. M. Ritter, P. K. Moore; Pharmacology, 5th Ed., Churchilll Livingstone, (2003).

- Himmel, H.M.Cardiovasc.DrugRev.1994, 12, 32-47.

- Naftopidil. Drugs Future.1987, 12, 31- 33.

- Yin-Xiang Sun, Bi-Yun Huang, Mu Yuan*, Jing-Shan Shi, Development of a chiral HPLC method for the analysis of Naftopidil enantiomers, Journal of Chinese Pharmaceutical Sciences, 2009 18 (1): 61-63 ISSN: 1003-1057 CN: 11-2863/R.

- YIN Yan-jie, ZHANG Qi-ming, TIAN Song- jiu,	Direct	Chromatography Separation of Six Pharmaceutical Racemates

on a AGP Chiral Stationary Phase, Chinese Pharmaceutical Journal, 2008-09.

- Yu DH, Wan LT, Lou YQ, Methodological study on the determination of Naftopidil concentration in biological samples by HPLC, Article in Chinese, 1995;30(4):286-90.

- Yuan M, Tang L, Fu X, Shi J, Zheng Z, Liu X, Determination of Naftopidil and its main metabolite in plasma by high-performance liquid chromatography, Article in Chinese, 1998 Oct;33(10):768-71.

- Ding Jinsong, Jiang Xuehua, Zhu Hao, Determination of Naftopidil in Dog Plasma by High-Performance Liquid Chromatography and Study on Its Pharmacokinetics, Journal of Chinese Pharmaceutical Sciences, 2000 9 (4): 200-203 ISSN: 1003-1057 CN: 11-2863/R.

- Jia Yun-Tao, Jiang Xue-hua, Ding Jin- Song, Pharmacokinetics of Naftopidil preparations in dogs, Central South Pharmacy 2003-02.

- J.A.Murillo Pulgarin, A.Alanon Molina, Maria Teresa Alanon Pardo, Direct determination of Naftopidil by non-protected fluid room temperature phosphorescence, Analyst, 126,234-238, 2001.

- J.A.Murillo Pulgarin, A.Alanon Molina, Maria Teresa Alanon Pardo, Non protected fluid room temperature phosphorimetric procedure for the direct determination of Naftopidil in biological fluids, Fresenius’ Journal of Analytical Chemistry; 371,903-910,2001.

- CH, Q2 (R1) Validation of Analytical Procedures: text and methodology; 2005.

- United States Pharmacopoeia, 28th Ed., Rockville, MD, United States Pharmacopoeia convention, inc., 2749 (2004).